MODULE Battery

! Procedure to query the camera and fetch center coordinates for a single item

PROC Battery_get_values()

MoveL Redy_Camara_Diamonds, v100, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

!        WaitTime 1;

!! Trigger camera to photo

send_string := input_string;

socket_start := TRUE;

startPLC := FALSE;

WaitTime 0.1;

Connect2;

!        WaitTime 1;

TPWrite "nval1:"\Num:=nval1;

TPWrite"for_cycle_times: "\Num:=for_cycle_times;

IF ((nval1 = 0 AND nval2 = 0) OR receive_string = ",;") THEN

TPWrite "Error: Receive string from Camera is wrong or target not found";

RETURN;

ENDIF

!!! Vision offset calculation

nval1 := nval1 + diamondX;  ! 蓝色 X 补偿

nval2 := nval2 + diamondY;  ! 蓝色 Y 补偿

nval3 := nval3 + diamondX;  ! 绿色 X 补偿

nval4 := nval4 + diamondY;  ! 绿色 Y 补偿

nval5 := nval5 + diamondX;  ! 黄色 X 补偿

nval6 := nval6 + diamondY;  ! 黄色 Y 补偿

MoveL Redy_Camara_Diamonds, v100, z0, tool_D\WObj:=wobj0;

ENDPROC

! Main execution routine calling all four colors sequentially

! 修改后的 Battery_work 逻辑

PROC Battery_work()

! 1. 先尝试获取多电池 (Blue, Green, Yellow)

input_string := "Getvalues";

Battery_get_values;

! 1. 判断是否为多电池 (Blue, Green, Yellow)

! 使用 <> (不等于) 替代 NOT，语法更清晰，编译器不会报错

IF (nval5 <> diamondX OR nval6 <> diamondY) AND (nval1 <> 0 OR nval2 <> 0) AND (receive_string <> ",;") THEN

TPWrite "检测到多电池坐标，自动执行任务二 (Blue, Green, Yellow)";

Battery_work_2;

RETURN;

ENDIF

! 2. 若多电池未检测到，尝试发送 Redvalues 获取红色电池坐标

input_string := "Redvalues";

Battery_get_values;

! 3. 判断是否检测到红色电池 (增加括号隔离字符串比较，确保不出错)

IF (nval1 = 0 AND nval2 = 0) OR (receive_string = ",;") THEN

TPWrite "视野内未检测到红色电池，跳过当前循环...";

RETURN;

ELSE

TPWrite "检测到单电池坐标，自动执行任务一 (Red)";

Battery_work_1;

ENDIF

input_string := "Redvalues";

Battery_get_values;

IF ((nval1 = 0 AND nval2 = 0) OR receive_string = ",;") THEN

TPWrite "视野内未检测到红色电池，跳过当前循环...";

RETURN;

ELSE

TPWrite "检测到单电池坐标，自动执行任务一 (Red)";

Battery_work_1;

ENDIF

ENDPROC

! Process RED Battery

PROC Battery_work_1()

VAR num positionX;

VAR num positionY;

VAR num positionZ;

VAR num positionR;

VAR num offSetX;

VAR num offSetY;

VAR num offSetZ;

VAR num offSetAngle;

!input_string := "Redvalues";

!Battery_get_values;

IF ((nval1 = 0 AND nval2 = 0) OR receive_string = ",;") THEN

TPWrite "Red battery not detected, skipping...";

RETURN;

ENDIF

TPWrite "Red-position: " + receive_string;

positionZ := D_pick.trans.z;

positionX := nval1;

positionY := nval2;

positionR := 0;

offSetX := nval1 - Redy_Camara_Diamonds.trans.x - 5.8 + 9;

offSetY := nval2 - Redy_Camara_Diamonds.trans.y - 0.5 + 55;

offSetZ := D_pick.trans.z - Redy_Camara_Diamonds.trans.z;

offSetAngle := 0;

! Move to center coordinates

MoveL RelTool (Redy_Camara_Diamonds, 0, 0, 0\Rz:= offSetAngle), v800, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)\Rz:= offSetAngle), v800, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+40)\Rz:= offSetAngle), v600, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ-6)\Rz:= offSetAngle), v400, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

! Pick up battery

WaitTime 1;

Set IIO_do2;

TPWrite "Finish pick Red battery";

WaitTime 0.3;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)), v800, z0, tool_D\WObj:=wobj0;

WaitTime 1.5;

MoveL RelTool(D_put_Red, 0, 0, -100), v800, z10, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

MoveL D_put_Red, v400, z0, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

WaitTime 1;

ReSet IIO_do2;

WaitTime 1.5;

TPWrite "Finish put Red battery";

MoveL Offs( Redy_Camara_Diamonds, 0, 0, 0), v400, z0, tool_D\WObj:=wobj0;

ENDPROC

! Process BLUE Battery

! Process Multiple Batteries (Yellow -> Green -> Blue)

PROC Battery_work_2()

VAR num positionX;

VAR num positionY;

VAR num positionZ;

VAR num positionR;

VAR num offSetX;

VAR num offSetY;

VAR num offSetZ;

VAR num offSetAngle;

! ===================================================

! 第一阶段：处理黄色电池 (抓取顺序 1，使用 nval5, nval6)

! 放置位置：原蓝色位置 (D_put_Blue)

! ===================================================

IF ((nval5 = 0 AND nval6 = 0) OR receive_string = ",;") THEN

TPWrite "Yellow battery not detected, skipping...";

ELSE

TPWrite "Yellow-position: " + receive_string;

positionZ := D_pick.trans.z;

positionX := nval5;

positionY := nval6;

positionR := 0;

! 保持黄色电池原本的抓取补偿参数

offSetX := nval5 - Redy_Camara_Diamonds.trans.x - 5.8 + 9;

offSetY := nval6 - Redy_Camara_Diamonds.trans.y - 0.5 + 52;

offSetZ := D_pick.trans.z - Redy_Camara_Diamonds.trans.z;

offSetAngle := 0;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)\Rz:= offSetAngle), v800, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+40)\Rz:= offSetAngle), v600, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ-5)\Rz:= offSetAngle), v400, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

! 抓取黄色电池

WaitTime 1;

Set IIO_do2;

TPWrite "Finish pick Yellow battery";

WaitTime 0.3;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)), v800, z0, tool_D\WObj:=wobj0;

WaitTime 0.5;

! 放置黄色电池 -> 更改为蓝色电池原有的放置点 D_put_Blue

MoveL RelTool(D_put_Yellow, 0, 0, -100), v600, z10, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

MoveL D_put_Yellow, v400, z0, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

WaitTime 1;

ReSet IIO_do2;

WaitTime 1.5;

TPWrite "Finish put Yellow battery at Blue position";

! 放置完抬高

MoveL RelTool(D_put_Yellow, 0, 0, -100), v800, z10, tool_D\WObj:=wobjDiamond;

ENDIF

! ===================================================

! 第二阶段：处理绿色电池 (抓取顺序 2，使用 nval3, nval4)

! 放置位置：保持不变 (D_put_Green)

! ===================================================

IF ((nval3 = 0 AND nval4 = 0) OR receive_string = ",;") THEN

TPWrite "Green battery not detected, skipping...";

ELSE

TPWrite "Green-position: " + receive_string;

positionZ := D_pick.trans.z;

positionX := nval3;

positionY := nval4;

positionR := 0;

offSetX := nval3 - Redy_Camara_Diamonds.trans.x - 5.8 - 10;

offSetY := nval4 - Redy_Camara_Diamonds.trans.y - 0.5 + 52;

offSetZ := D_pick.trans.z - Redy_Camara_Diamonds.trans.z;

offSetAngle := 0;

! 从上一个放置点飞往绿色电池抓取预备点

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)\Rz:= offSetAngle), v800, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+40)\Rz:= offSetAngle), v600, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ-5)\Rz:= offSetAngle), v400, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

WaitTime 1;

Set IIO_do2;

TPWrite "Finish pick Green battery";

WaitTime 0.3;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)), v800, z0, tool_D\WObj:=wobj0;

WaitTime 0.5;

MoveL RelTool(D_put_Green, 0, 0, -100), v600, z10, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

MoveL D_put_Green, v400, z0, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

WaitTime 1;

ReSet IIO_do2;

WaitTime 1.5;

TPWrite "Finish put Green battery";

MoveL RelTool(D_put_Green, 0, 0, -100), v800, z10, tool_D\WObj:=wobjDiamond;

ENDIF

! ===================================================

! 第三阶段：处理蓝色电池 (抓取顺序 3，使用 nval1, nval2)

! 放置位置：原黄色位置 (D_put_Yellow)

! ===================================================

IF ((nval1 = 0 AND nval2 = 0) OR receive_string = ",;") THEN

TPWrite "Blue battery not detected, skipping...";

ELSE

TPWrite "Blue-position: " + receive_string;

positionZ := D_pick.trans.z;

positionX := nval1;

positionY := nval2;

positionR := 0;

! 保持蓝色电池原本的抓取补偿参数

offSetX := nval1 - Redy_Camara_Diamonds.trans.x - 5.8 - 9;

offSetY := nval2 - Redy_Camara_Diamonds.trans.y - 0.5 + 60;

offSetZ := D_pick.trans.z - Redy_Camara_Diamonds.trans.z;

offSetAngle := 0;

! 从绿色放置点上方飞往蓝色电池抓取预备点

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)\Rz:= offSetAngle), v800, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+40)\Rz:= offSetAngle), v600, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ-5)\Rz:= offSetAngle), v400, z0, tool_D\WObj:=wobj0;

WaitRob\InPos;

WaitTime 1;

Set IIO_do2;

TPWrite "Finish pick Blue battery";

WaitTime 0.3;

MoveL RelTool (Redy_Camara_Diamonds, -offSetX, offSetY, - (offSetZ+100)), v800, z0, tool_D\WObj:=wobj0;

WaitTime 0.5;

! 放置蓝色电池 -> 更改为黄色电池原有的放置点 D_put_Yellow

MoveL RelTool(D_put_Blue, 0, 0, -100), v600, z10, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

MoveL D_put_Blue, v400, z0, tool_D\WObj:=wobjDiamond;

WaitRob\InPos;

WaitTime 1;

ReSet IIO_do2;

WaitTime 1.5;

TPWrite "Finish put Blue battery at Yellow position";

MoveL RelTool(D_put_Blue, 0, 0, -100), v800, z10, tool_D\WObj:=wobjDiamond;

ENDIF

! ===================================================

! 第四阶段：所有 3 种颜色全部抓放完成，回到拍照原点

! ===================================================

MoveL Offs( Redy_Camara_Diamonds, 0, 0, 0), v800, z0, tool_D\WObj:=wobj0;

ENDPROC

ENDMODULE